COMPUTERIZED VCR MULTI FUEL ENGINE WITH ACCESSORIES:

Technical Specifications:

| Engine: | Single cylinder, 4 stroke, (Computerized) VCR Engine |
| --- | --- |
| Cooling: | Water cooled |
| Power: | Diesel: (3.5 kW) Petrol: (4.5 kW) |
| Rated Speed: | 1600-1800 rpm |
| Bore: | 87.5 mm |
| Stroke: | 110 mm |
| Swept Volume: | 661 cc |
| Engine Start: | Electric Start with Battery & Alternator for charging the battery |
| Compression Ratio for Diesel mode: | 12:1 to 18:1 |
| Compression Ratio for petrol Mode: | 6:1 to 10:1 |
| Diesel mode: | 3.5 kW, 1600 - 1800 rpm, CR range 12-18. Injection variation:0- 25˚ BTDC |
| Petrol mode: | 4.5 kW@ 1800 rpm, Speed range 1000-1800 rpm, CR range 6-10 |
| Dynamometer: | Swinging Field Eddy current dynamometer, Water cooled, Capacity 7.5 kW with electronic controller |
| Propeller shaft: | With universal joints |
| Air box: | M.S fabricated with orifice meter and manometer |
| Fuel tank: | Capacity 10 lit, Type: Duel compartment, for Petrol & Diesel |
| Exhaust Gas Calorimeter: | Pipe in Pipe Type |
| Combustion Pressure Sensor: | Piezo sensor, Range 100 bar with low noise cable (Make PCB or equivalent imported) |
| Crank Angle Sensor: | Resolution 1 Deg, Speed 5500 RPM with TDC pulse |
| Data Acquisition System: | NI USB-6210, 16-bit, 250kS/s. (Make National instruments, USA) |
| Temperature Sensor: | Type RTD, PT100 for ambient and Thermocouple, Type K for Exhaust gas & water |
| Temperature transmitter: | Type two wire, Input RTD PT100, Range 0–100 Deg C, Output 0-10 V DC |
| Load Sensor: | Load Cell, Strain Gauge Type Range 0-50 Kg for Torque Measurement. |
| Fuel Flow Transmitter: | Gravimetric Fuel Flow transmitter, Resolution 0.1 g |
| Air Flow Transmitter: | Pressure transmitter, Range 500 mm WC |
| Software: | Engine performance analysis software or any other Engine performance analysis software. Windows based software developed in LAB View for - Data Logging - Real-Time Display - Tabulated & Graphical Results |
| Rota meter: | Engine cooling & Calorimeter cooling, Range 60-600 LPH |

- Designed for comprehensive analysis of IC engine performance under variable

load conditions & at different Compression ratio settings.

- The Engine can be used as both Petrol & Diesel engine.

- Battery assisted Electric start arrangement for engine & alternator for battery

charging

- High Speed real time data acquisition system with Windows based software.

EXPERIMENTAL CAPABILITIES

- Determination of Brake Power & Mechanical Efficiency at different load conditions.

- To study effect of change in compression ratio on Engine performance.

- Determination of Fuel consumption & Brake specific fuel consumption, Brake Thermal Efficiency.

- Determination of air consumption & volumetric efficiency, Air/Fuel Ratios.

- Calculation of Heat balance of the engine.

- To study P-V Diagram & P- diagram for Petrol / Diesel engine.

-o0o-